| UTC Project Information – Project 2.11 |  |
| --- | --- |
| Project Title | Culvert Rehabilitation using 3D Printed Diffusers |
| University | University of Maine |
| Principal Investigator | Roberto Lopez-Anido |
| PI Contact Information | rla@maine.edu |
| Co-PI(s) | Sunil Bhandari |
| Co-PI Contact Information | sunil.bhandari@maine.edu |
| Funding Source(s) and Amounts Provided | UMaine (Phases I and II + Phase III) : $158,467+ $142,566 MaineDOT(Phase I): $6,500 NHDOT (Phase III) : $20,000 |
| Total Project Cost | $327,533 |
| Agency ID or Contract No | 69A3551847101 |
| Start and End Dates | 07-01-2020 to 06-30-2022 |
| Brief Description of Research Project | What are culvert diffusers and why are they used? - Culverts are installed in highways to control water flows - Culverts deteriorate with age and need repairs. Current culvert retrofit technology: - Use slip-liners - Problem: reduction of the cross-section and hence the capacity of the culvert. New culvert outlet diffusers technology: - Increase the culvert capacity and compensate for the losses due to liner retrofit - Reduce outlet flow velocity and minimize erosion. Project objectives: - Explore the feasibility of using large scale 3D printing to manufacture culvert outlet diffusers with site-specific optimized designs. - Design and manufacture an experimental 3D printed culvert diffuser prototype to evaluate hydraulic capacity at the site - Assess the use of bio-based renewable polymer composites for manufacturing culvert diffusers. - Explore segmental manufacturing of 3D printed culvert diffusers. - Evaluate the performance of 3D printed culvert diffuser in the field. |
| Describe Implementation of Research Outcomes (or why not implemented) | 3D printed culvert outlet diffuser segments.; Assembled culvert diffuser segments. |
| Impacts/Benefits of Implementation (actual, not anticipated) | - Design of the diffuser is unique depending on the culvert and the site conditions. - Significant cost reduction to current manufacturing methods. - 3D printing is more cost effective for larger parts. - Segmental manufacturing of long and narrow parts enables faster manufacturing times. - Optimized designs of culvert diffusers based on site-specific hydraulic conditions can be manufactured using large-scale 3D printing at lower cost than current methods. - Hydraulic performance evaluation in the field, as well as ease of installation and durability have to be demonstrated for technology transfer. |
| Web Links - Reports - Project website | Bhandari, S., Lopez-Anido, R.A., Anderson, J. and Mann, A. “Large-scale extrusion-based 3D printing for highway culvert rehabilitation,” ANTEC 2021, Society of Plastic Engineers, Hybrid Edition, Mar. 22-23, 2021. |